B2315BT - Bluetooth Broadcast Protocol

Data structure:

| Serial number | Field name | Description |
| --- | --- | --- |
| 0 | Data type (0x09) + Content length (1 byte) | 1 octet 0x08——8 bytes |
| 1 | Data type (0x09) | 1 octet |
| 2 | (0x09 field) content | (7 bytes) 47323331304244——-(ASCAll code table conversion):G2310BD |
| 3 | Data type (0xFF) + user data length (xx) | 1 octet, data length occupied by <Manufacturer Specific Data> |
| 4 | Data type (0xFF) | 1 octet, Manufacturer Specific Data |
| 5 | Company ID (0x4D5A) | 2 octets fixed value, Company ID (MZ) |
| 6 | User data - byte 0 (Format) | Fixed value 0x4D |
| 7 | User data - byte 1 (CMDID) | Command Word (0x0D) |
| 8 | User data-Byte 2 (Version) | Software version |
| 9 | User data-Byte 3 (adv number) | Broadcast count adv number, if the data is updated, then +1, added to 255, then start from 0 |
| 10 | User data-Byte 4 | bit[0:3]: Reserved, bit[4:7]: Battery level in percentage, range 0-10 |
| 11 | User data-Byte 5 (Status) | bit0-1 bits: Reserved, default value is 0, bit2 bits: Reserved, bit3 bits: Reserved, bit4 bits: SOS status: 1 alarm 0 normal, bit5 bits: Wearing status: 1 not wearing 0 wearing, bit6 bits: Motion status: 1 moving 0 static |
| 12 | User data-Byte 6 (Heart Rate) | Heart rate (BPM): Below 200 is normal heart rate; 252 measurement abnormal; 250 not worn, 251 heart rate module abnormal |
| 13 | User data - Byte 7 (Body Temperature) | Body surface temperature (converted to decimal value + 200) / 10; for example, 156 represents 35.6 degrees Celsius 0xAA |
| 14 | User data - Byte 8 (Step Count) | Step count |
| 15 | User data - Byte 10 (Blood Pressure Systolic) | Systolic blood pressure, sbp |
| 16 | User data - Byte 11 (Blood Pressure Diastolic) | Diastolic blood pressure dbp |
| 17 | User data - Byte 12 (Sleep) | Current status. 0x00: Not detected, 0x01: Awake, 0x02: Light sleep, 0x03: Deep sleep |
| 18 | User data-byte 13 | CheckSum checksum (checksum of userdata0 to userdata 12) |

Example:

02010608094732333130424411FF4D5A4D0D01C990004CA75600764A0164

020106:Beginning of the message, meaningless

08:Data type (0x09) + content length, 08---> 16 hexadecimal to 10 decimal 8---length is 8 bytes

09:Data type 0x09

47323331304244:ASCAll code table converted to string-->G2310BD

11:Data type (0xFF) + user data length, 0x11---> 16 hexadecimal to 10 decimal 17---length is 17 bytes

FF:Data type (0xFF)

4D5A:Company ID Vendor ID fixed value

4D:User data-bytes0(Format) Fixed value

0D:User data-bytes1(CMDID) Fixed value

01:User data-bytes2 (Version) Software version 01

C9:User data-bytes3(adv number) Broadcast count C9-->16hexadecimal to 10decimal 201---broadcast count is 201

90: User data-bytes4  90-->16hexadecimal to binary 10010000  Battery level is bit[4-7]:1001--->binary to decimal 9--battery level is 90%

00: User data-bytes5(Status) 00-->16hexadecimal to binary00000000  bit0-bit3:0,bit4: SOS status: 0normal, bit5: Wearing status:0wearing, bit6 bit: Motion status:0static

4C: User data-bytes6(Heart rate) 4C-->16hexadecimal to decimal 76---heart rate is 76

A7:User data-bytes7(Body temperature) a716hexadecimal to 10decimal 167-->(167+200)/10=36.7--body temperature is 36.7

5600:User data-bytes8(Step count) converted to big-endian-->0056-->16hexadecimal to decimal--->86---step count is 86 steps

76:User data-bytes10(Blood pressure Systolic pressure)76-->16hexadecimal to decimal 118--blood pressure Systolic pressure is 118

4A:User data-bytes11(Blood pressure diastolic pressure)4A-->16Hexadecimal to decimal74--Blood pressure diastolic pressure is74

01:User data-byte12(Sleep)01---Sleep state Awake

64:User data-byte13 Check code