B2315P - Bluetooth Broadcast Protocol

Data structure:

| Serial number | Field name | Description |
| --- | --- | --- |
| 0 | Data type (0x09) + Content length (1 byte) | 1 octet 0x08——8 bytes |
| 1 | Data type (0x09) | 1 octet |
| 2 | (0x09 field) content | (7 bytes) 47323331304244——-(ASCAll code table conversion):G2310BD |
| 3 | Data type (0xFF) + user data length (xx) | 1 octet, data length occupied by <Manufacturer Specific Data> |
| 4 | Data type (0xFF) | 1 octet, Manufacturer Specific Data |
| 5 | Company ID (0x4D5A) | 2 octets fixed value, Company ID (MZ) |
| 6 | User data - byte 0 (Format) | Fixed value 0x4D |
| 7 | User data - byte 1 (CMDID) | Command Word (0x0D) |
| 8 | User data - Byte 2 (Version) | Software version |
| 9 | User data - Byte 3 (adv number) | Broadcast count adv number, if the data is updated, then +1, added to 255, then start from 0 |
| 10 | User data - Byte 4 | bit[0:3]: Reserved, bit[4:7]: Battery level in percentage, range 0-10 |
| 11 | User data - Byte 5 (Status) | bit0-1 bits: Reserved, default value is 0, bit2 bits: Reserved, bit3 bits: Reserved, bit4 bits: SOS status: 1 alarm 0 normal, bit5 bits: Wearing status: 1 not wearing 0 wearing, bit6 bits: Motion status: 1 moving 0 static |
| 12 | User data - Byte 6 (Heart Rate) | Heart rate (BPM): Below 200 is normal heart rate; 252 measurement abnormal; 250 not worn, 251 heart rate module abnormal |
| 13 | User data - Byte 7 (body temperature) | Body surface temperature (converted to decimal value + 200) / 10; for example, 156 represents 35.6 degrees Celsius 0xAA |
| 14 | User data - Byte 8 (step count) | Step count |
| 15 | User data - Byte 10 (blood pressure, systolic blood pressure) | Systolic blood pressure, sbp |
| 16 | User data - Byte 11 (blood pressure, diastolic blood pressure) | Diastolic blood pressure dbp |
| 17 | User data - Byte 12 (blood pressure, diastolic blood pressure) | Diastolic blood pressure dbp |
| 18 | User data - Byte 13 (blood oxygen) | Blood Oxygen |
| 19 | User data-Byte 13 | CheckSum (checksum of userdata0 to userdata 12) |

Example:

02010608094732333130424411FF4D5A4D0D01C990004CA75600764A630164

020106: Start of message, meaningless

08: Data type (0x09) + content length, 08---> 16 hexadecimal to 10 decimal 8---length is 8 bytes

09: Data type 0x09

47323331304244: ASCAll code table converted to string-->G2310BD

11: Data type (0xFF) + user data length, 0x11---> 16 hexadecimal to 10 decimal 17---length is 17 bytes

FF: Data type (0xFF)

4D5A: Company ID Vendor ID fixed value

4D: User data - byte 0 (Format) Fixed value

0D: User data - byte 1 (CMDID) Fixed value

01: User data - byte 2 (Version) Software version 01

C9: User data - byte 3 (adv number) Broadcast count C 9 --> 16 Hexadecimal to 10 Decimal 201 --- Broadcast count is 201

90: User data - byte 4 90 --> 16 Hexadecimal to binary 10010000 Power is bit[4 - 7]: 1001 ---> Binary to decimal 9 -- Power is 90%

00: User data - byte 5 (Status) 00 --> 16 Hexadecimal to binary 00000000 bit 0 - bit 3: 0, bit 4: SOS status: 0 normal, bit 5: Wearing status: 0 wearing, bit 6: Motion status: 0 static

4C: User data - byte 6 (Heart rate) 4 C --> 16 Hexadecimal to decimal 76 --- Heart rate is 76

A7: User data - byte 7 (Body temperature) a 716 Hexadecimal to 10 Decimal 167 --> (167 + 200) / 10 = 36 . 7 -- Body temperature is 36 . 7

5600: User data - byte 8 (Step count) converted to big-endian --> 0056 --> 16 Hexadecimal to decimal ---> 86 --- Step count is 86 steps

76:User data-bytes10(Blood pressure systolic)76-->16Hexadecimal to decimal  118--Blood pressure systolic is118

4A:User data-bytes11(Blood pressure diastolic pressure)4A-->16Hexadecimal to decimal74--Blood pressure diastolic pressure is74

63:User data-byte 12 (blood oxygen) 63-->16 hexadecimal to decimal 99--blood oxygen is 99%

01:User data-byte13(Sleep)01---Sleep state awake

64:User data-byte14 Check code