B601 - Bluetooth Broadcast Protocol

Data structure:

| Serial number | Field name | Description |
| --- | --- | --- |
| 0 | Data type (0x09) + Content length (1 byte) | 1 octet 0x08——8 bytes |
| 1 | Data type (0x09) | 1 octet |
| 2 | (0x09 field) content | (7 bytes) 47323331304244——-(ASCAll code table conversion):G2310BD |
| 3 | Data type (0xFF) + user data length (xx) | 1 octet, data length occupied by <Manufacturer Specific Data> |
| 4 | Data type (0xFF) | 1 octet, Manufacturer Specific Data |
| 5 | Company ID (0x4D5A) | 2 octets fixed value, Company ID (MZ) |
| 6 | User data - byte 0 (Format) | Fixed value 0x4D |
| 7 | User data - byte 1 (CMDID) | Command Word (0x0D) |
| 8 | User data-Byte 2 (Version) | Software version |
| 9 | User data-Byte 3 (adv number) | Broadcast count adv number, if the data is updated, then +1, added to 255, then start from 0 |
| 10 | User data-Byte 4 | bit[0:3]: Reserved, bit[4:7]: Battery level in percentage, range 0-10 |
| 11 | User data-Byte 5 (Status) | bit0-1 bits: Reserved, default value is 0, bit2 bits: Reserved, bit3 bits: Reserved, bit4 bits: SOS status: 1 alarm 0 normal, bit5 bits: Wearing status: 1 not wearing 0 wearing, bit6 bits: Motion status: 1 moving 0 static |
| 12 | User data-Byte 6 (Heart Rate) | Heart rate (BPM): Below 200 is normal heart rate; 252 measurement abnormal; 250 not worn, 251 heart rate module abnormal |
| 13 | User data - Byte 7 (Body Temperature) | Body surface temperature (converted to decimal value + 200) / 10; for example, 156 represents 35.6 degrees Celsius 0xAA |
| 14 | User data - Byte 8 (Step Count) | Step count |
| 15 | User data - Byte 10 (Blood Pressure Systolic) | Systolic blood pressure, sbp |
| 16 | User data - Byte 11 (Blood Pressure Diastolic) | Diastolic blood pressure dbp |
| 17 | User data - Byte 12 (Blood Oxygen) | Blood oxygen data |
| 18 | User data - Byte 13 (Sleep) | Current status. 0x00: Not detected, 0x01: Awake, 0x02: Light sleep, 0x03: Deep sleep |
| 19 | User data - byte 14 | CheckSum checksum (checksum of userdata0 to userdata 13) |

Example:

Note:

Data is0when not worn or when health data is not detected.

B601 can be connected to the broadcast version name at startup, and cannot be connected after1minute

02010608094732333130424411FF4D5A4D0D01C990004CA75600764A0164

020106: Start of message, meaningless

08: Data type (0x09) + content length,08--->Hexadecimal to decimal 8---length is 8 bytes

09: Data Type 0x09

47323331304244: Convert ASCAll code table to string -->G2310BD

11: Data type (0xFF) + user data length, 0x11 ---> Hexadecimal to decimal 17 --- Length is 17 bytes

FF: Data type (0xFF)

4 D5A: Company ID Vendor ID fixed value

4 D: User data-byte 0 (Format) Fixed value

0 D: User data-byte 1 (CMDID) Fixed value

01: User data-byte 2 (Version) Software version 01

C9: User data-byte 3 (adv number) Broadcast count C9 --> Hexadecimal to decimal 201 --- Broadcast count is 201

90: User data-byte 4 90 --> Hexadecimal to binary 10010000 Power is bit[4-7]: 1001---> Binary to decimal 9--Power is 90%

00: User data-byte5 (Status) 00-->Hexadecimal to binary 00000000 bit0-bit3:0, bit4: SOS status: 0 normal, bit5: Wearing status: 0 wearing, bit6: Motion status: 0 static

4C: User data-byte6 (Heart Rate) 4C-->Hex to Decimal 76---Heart Rate is 76

A7:User data-byte7(Body Temperature) a7 Hex to10 Decimal 167-->(167+200)/10=36.7--Body Temperature is 36.7

5600:User data-byte8(Step Count) Convert to Big Endian-->0056-->Hex to Decimal--->86---Step Count is 86 steps

76:User data-byte10(Blood Pressure Systolic)76-->Hex to Decimal 118--Blood Pressure Systolic is 118

4A:User data-byte11(Blood Pressure Diastolic)4A-->Hex to Decimal 74--Blood Pressure Diastolic is 74

63:User data-Byte 12 (Oxygen saturation) 63-->hexadecimal to decimal conversion 99--Oxygen saturation is 99%

01:User data-byte13(Sleep)01---Sleep State Awake

64:User data-byte14 Check Code