BS01 Wristband BLE connect Mode Protocol

| Type | description | uuid | method | length | value | default |
| --- | --- | --- | --- | --- | --- | --- |
| Service1 | Peripheral control | 0000190A-0001-1000-8000-00805F9B34FB |  |  |  |  |
| characteristic | turn off the unit function | 00000001-0001-1000-8000-00805F9B34FB | Write/Read | 1 byte | 1: on 0：off | 1 |
| characteristic | LED indicator | 00000002-0001-1000-8000-00805F9B34FB | Write/Read | 1 byte | 1: on 0：off | 1 |
| characteristic | READ batterydata | 00000003-0001-1000-8000-00805F9B34FB | Read | 1byte | 0~100 percent | Last read sensor value |
| characteristic | powe-saver mode Enable/disable function | 0000004-0001-1000-8000-00805F9B34FB | Write/Read | 1 byte | 1: on 0:off | 0 |
| characteristic | broadcast interval Update rate | 0000005-0001-1000-8000-00805F9B34FB | Write/Read | 1 byte | Unit(second) | 20 |
| characteristic | Set blePairing code | 0000006-0001-1000-8000-00805F9B34FB | Write/Read | 3 bytes |  | 000000 |
| characteristic | Enable Pairing code | 0000007-0001-1000-8000-00805F9B34FB | Write/Read | 1 byte | 1: on 0: off | 0 |
| characteristic | Enable encrypt | 0000008-0001-1000-8000-00805F9B34FB | Write/Read | 1bytes | 1: on 0: off | 0 |
| characteristic | Set encrypt key | 0000009-0001-1000-8000-00805F9B34FB | Write/Read | 1 byte |  | 0xFE |
| Service2 | HRS | 0000190A-0002-1000-8000-00805F9B34FB |  |  |  |  |
| characteristic | Enable/disable function | 0000001-0002-1000-8000-00805F9B34FB | Write/Read | 1 byte | 1: on 0：off | 0 |
| characteristic | HRS Read data | 00000002-0002-1000-8000-00805F9B34FB | Notify/Read | 1byte |  | Last read sensor value |
| characteristic | Update frequency | 00000003-0002-1000-8000-00805F9B34FB | Write/Read | 2bytes | Unit(second) | 20 |
| Service3 | accelerometer | 0000190A-0003-1000-8000-00805F9B34FB |  |  |  |  |
| characteristic | accelerometer Enable/disable function | 0000001-0003-1000-8000-00805F9B34FB | Write/Read | 1 byte | 1: on 0：off | 0 |
| characteristic | read(x,y,z-positions) Read data | 00000002-0003-1000-8000-00805F9B34FB | Notify/Read | 6bytes |  | Last read sensor value |
| characteristic | Update frequency | 00000003-0003-1000-8000-00805F9B34FB | Write/Read | 2bytes | Unit(second) | 20 |
| Service4 | Skin&Internal temperature | 0000190A-0004-1000-8000-00805F9B34FB |  |  |  |  |
| characteristic | skin&Internaltemperatue Enable/disable function | 0000001-0004-1000-8000-00805F9B34FB | Write/Read | 1 byte | 1: on 0：off | 0 |
| characteristic | Read Temperature in C Read data | 00000002-0004-1000-8000-00805F9B34FB | Notify/Read | 4 bytes |  | Last read sensor value |
| characteristic | Update frequency | 00000003-0004-1000-8000-00805F9B34FB | Write/Read | 2bytes | Unit(min) | 30 |
| Service5 | Sp02 | 0000190A-0005-1000-8000-00805F9B34FB |  |  |  |  |
| characteristic | Sp02 Enable/disable function | 0000001-0005-1000-8000-00805F9B34FB | Write/Read | 1 byte | 1: on 0：off | 0 |
| characteristic | Read Sp02data | 00000002-0005-1000-8000-00805F9B34FB | Notify/Read | 1byte |  | Last read sensor value |
| characteristic | Update frequency | 00000003-0005-1000-8000-00805F9B34FB | Write/Read | 1 byte | Unit(min) | 30 |
| Service6 | HRV | 0000190A-0006-1000-8000-00805F9B34FB |  |  |  |  |
| characteristic | HRV Enable/disable function | 0000001-0006-1000-8000-00805F9B34FB | Write/Read | 1 byte | 1: on 0：off | 0 |
| characteristic | Read HRVdata | 00000002-0006-1000-8000-00805F9B34FB | Notify/Read | 2byte |  | Last read sensor value |
| characteristic | Update frequency | 00000003-0006-1000-8000-00805F9B34FB | Write/Read | 1 byte | Unit(seconds) | 30 |

手环操作和灯闪烁的含义：

手环有佩戴检测，不佩戴就进入低功耗模式。

1.手环有一个RGB灯和一个蓝色LED灯

2.当手环和其他设备连接，蓝色LED灯会亮起。断开，蓝色LED灯熄灭

3.手环充电时，RGB灯会循环闪烁

4.手环刚开机，RGB灯循环闪烁两次。开机一分钟后，RGB灯作为电量指示

电量》=80%	绿灯亮

电量》=20%	黄灯亮（红+绿）

电量》=10%	红灯亮

电量《10%	红灯闪烁